Part-1 : Design and Implementation of the FLC

% Create a new fuzzy inference system

fis = mamfis('Name', 'IntelligentAssistiveCare');

fis = addInput(fis, [10 35], 'Name', 'Temperature');

fis = addMF(fis, 'Temperature', 'trapmf', [10 10 15 18], 'Name', 'VeryCold');

fis = addMF(fis, 'Temperature', 'trimf', [15 18 21], 'Name', 'Cold');

fis = addMF(fis, 'Temperature', 'trimf', [18 21 24], 'Name', 'Comfortable');

fis = addMF(fis, 'Temperature', 'trimf', [21 24 27], 'Name', 'Warm');

fis = addMF(fis, 'Temperature', 'trapmf', [24 27 35 35], 'Name', 'Hot');

fis = addInput(fis, [0 100], 'Name', 'Humidity');

fis = addMF(fis, 'Humidity', 'trapmf', [0 0 30 45], 'Name', 'Low');

fis = addMF(fis, 'Humidity', 'trimf', [30 50 70], 'Name', 'Medium');

fis = addMF(fis, 'Humidity', 'trapmf', [55 70 100 100], 'Name', 'High');

fis = addInput(fis, [0 100], 'Name', 'LightLevel');

fis = addMF(fis, 'LightLevel', 'trapmf', [0 0 20 40], 'Name', 'Dark');

fis = addMF(fis, 'LightLevel', 'trimf', [20 50 80], 'Name', 'Dim');

fis = addMF(fis, 'LightLevel', 'trapmf', [60 80 100 100], 'Name', 'Bright');

fis = addInput(fis, [0 24], 'Name', 'TimeOfDay');

fis = addMF(fis, 'TimeOfDay', 'trapmf', [0 0 5 7], 'Name', 'Night');

fis = addMF(fis, 'TimeOfDay', 'trimf', [5 12 17], 'Name', 'Day');

fis = addMF(fis, 'TimeOfDay', 'trapmf', [17 19 24 24], 'Name', 'Evening');

fis = addInput(fis, [0 10], 'Name', 'ActivityLevel');

fis = addMF(fis, 'ActivityLevel', 'trapmf', [0 0 2 4], 'Name', 'Low');

fis = addMF(fis, 'ActivityLevel', 'trimf', [2 5 8], 'Name', 'Medium');

fis = addMF(fis, 'ActivityLevel', 'trapmf', [6 8 10 10], 'Name', 'High');

fis = addOutput(fis, [-2 2], 'Name', 'HVACControl');

fis = addMF(fis, 'HVACControl', 'trapmf', [-2 -2 -1.5 -1], 'Name', 'CoolStrongly');

fis = addMF(fis, 'HVACControl', 'trimf', [-1.5 -1 -0.5], 'Name', 'CoolMildly');

fis = addMF(fis, 'HVACControl', 'trimf', [-0.5 0 0.5], 'Name', 'Maintain');

fis = addMF(fis, 'HVACControl', 'trimf', [0.5 1 1.5], 'Name', 'HeatMildly');

fis = addMF(fis, 'HVACControl', 'trapmf', [1 1.5 2 2], 'Name', 'HeatStrongly');

fis = addOutput(fis, [0 1], 'Name', 'LightControl');

fis = addMF(fis, 'LightControl', 'trapmf', [0 0 0.2 0.4], 'Name', 'Off');

fis = addMF(fis, 'LightControl', 'trimf', [0.2 0.5 0.8], 'Name', 'Dim');

fis = addMF(fis, 'LightControl', 'trapmf', [0.6 0.8 1 1], 'Name', 'Bright');

fis = addOutput(fis, [0 1], 'Name', 'BlindControl');

fis = addMF(fis, 'BlindControl', 'trapmf', [0 0 0.2 0.4], 'Name', 'Closed');

fis = addMF(fis, 'BlindControl', 'trimf', [0.2 0.5 0.8], 'Name', 'PartiallyOpen');

fis = addMF(fis, 'BlindControl', 'trapmf', [0.6 0.8 1 1], 'Name', 'Open');

fis = addOutput(fis, [0 2], 'Name', 'PowerManagement');

fis = addMF(fis, 'PowerManagement', 'trapmf', [0 0 0.5 1], 'Name', 'Off');

fis = addMF(fis, 'PowerManagement', 'trimf', [0.5 1 1.5], 'Name', 'Standby');

fis = addMF(fis, 'PowerManagement', 'trapmf', [1 1.5 2 2], 'Name', 'On');

rules = [

"Temperature==VeryCold => HVACControl=HeatStrongly"

"Temperature==Cold & Humidity==Low => HVACControl=HeatMildly"

"Temperature==Comfortable & Humidity==Medium => HVACControl=Maintain"

"Temperature==Warm & Humidity==High => HVACControl=CoolMildly"

"Temperature==Hot => HVACControl=CoolStrongly"

"LightLevel==Dark & TimeOfDay==Night => LightControl=Off"

"LightLevel==Dim & TimeOfDay==Evening => LightControl=Dim"

"LightLevel==Bright & TimeOfDay==Day => LightControl=Bright"

"ActivityLevel==Low & TimeOfDay==Night => PowerManagement=Off"

"ActivityLevel==Medium & TimeOfDay==Day => PowerManagement=Standby"

"ActivityLevel==High & TimeOfDay==Day => PowerManagement=On"

"LightLevel==Bright => BlindControl=Closed"

"LightLevel==Dim => BlindControl=PartiallyOpen"

"LightLevel==Dark => BlindControl=Open"

];

fis = addRule(fis, rules);

% Sample input values

input = [22, 60, 50, 14, 5]; % [Temperature, Humidity, LightLevel, TimeOfDay, ActivityLevel]

% Evaluate the FIS

output = evalfis(fis, input);

% Display the result

disp(['HVAC Control Output: ', num2str(output(1))]);

disp(['Light Control Output: ', num2str(output(2))]);

disp(['Blind Control Output: ', num2str(output(3))]);

disp(['Power Management Output: ', num2str(output(4))]);

% Plot membership functions for Temperature

figure;

plotmf(fis, 'input', 1);

title('Temperature Membership Functions');

% Plot membership functions for Humidity

figure;

plotmf(fis, 'input', 2);

title('Humidity Membership Functions');

% Plot membership functions for Light Level

figure;

plotmf(fis, 'input', 3);

title('Light Level Membership Functions');

% Plot membership functions for Time of Day

figure;

plotmf(fis, 'input', 4);

title('Time of Day Membership Functions');

% Plot membership functions for Activity Level

figure;

plotmf(fis, 'input', 5);

title('Activity Level Membership Functions');

% Plot membership functions for HVAC Control

figure;

plotmf(fis, 'output', 1);

title('HVAC Control Membership Functions');

% Plot membership functions for Light Control

figure;

plotmf(fis, 'output', 2);

title('Light Control Membership Functions');

% Plot membership functions for Blind Control

figure;

plotmf(fis, 'output', 3);

title('Blind Control Membership Functions');

% Plot membership functions for Power Management

figure;

plotmf(fis, 'output', 4);

title('Power Management Membership Functions');

% Show the rule viewer

figure;

ruleview(fis);

% Show the surface viewer

figure;

gensurf(fis);

% Create a range of temperature and humidity values

temp_range = 10:0.5:35;

humidity_range = 0:2:100;

% Initialize output matrix

output_matrix = zeros(length(temp_range), length(humidity_range));

% Evaluate FIS for each combination of temperature and humidity

for i = 1:length(temp_range)

for j = 1:length(humidity_range)

input = [temp_range(i), humidity_range(j), 50, 12, 5]; % Fixed values for other inputs

output_matrix(i,j) = evalfis(fis, input);

end

end

% Plot control surface

figure;

surf(humidity_range, temp_range, output_matrix);

xlabel('Humidity (%)');

ylabel('Temperature (°C)');

zlabel('HVAC Control');

title('Control Surface: HVAC Control');

colorbar;

% Analyze specific scenarios

scenarios = [

15, 30;  % Cold and dry

22, 50;  % Comfortable

28, 70;  % Warm and humid

];

figure;

for i = 1:size(scenarios, 1)

input = [scenarios(i,1), scenarios(i,2), 50, 12, 5]; % Fixed values for other inputs

subplot(size(scenarios, 1), 1, i);

[output, mfGrades] = evalfis(fis, input, 'OutputFormat', 'Detailed');

% Plot rule activation

bar(mfGrades.rule);

title(sprintf('Rule Activation (Temp: %.1f°C, Humidity: %.1f%%)', scenarios(i,1), scenarios(i,2)));

xlabel('Rule Number');

ylabel('Activation Level');

ylim([0 1]);

% Display HVAC control output

text(0.5, 0.9, sprintf('HVAC Control: %.2f', output), 'Units', 'normalized', 'HorizontalAlignment', 'center');

end

% Analyze controller behavior over time

time = 0:23; % 24-hour period

temp_profile = 15 + 10 * sin((time - 6) * pi / 12); % Temperature profile

humidity_profile = 50 + 20 * sin((time - 3) * pi / 12); % Humidity profile

hvac_output = zeros(size(time));

for i = 1:length(time)

input = [temp_profile(i), humidity_profile(i), 50, time(i), 5]; % Fixed values for other inputs

hvac_output(i) = evalfis(fis, input);

end

figure;

subplot(2,1,1);

plot(time, temp_profile, 'r-', time, humidity_profile, 'b--');

xlabel('Time (hours)');

ylabel('Value');

legend('Temperature (°C)', 'Humidity (%)');

title('Temperature and Humidity Profile over 24 hours');

subplot(2,1,2);

plot(time, hvac_output, 'g-');

xlabel('Time (hours)');

ylabel('HVAC Control');

title('HVAC Control Output over 24 hours');

ylim([-2 2]);

Task 2: Compare different optimization techniques on CEC’2005 functions.

% Benchmark Functions Comparison

% Define test functions

functions = {@(x) sum(x.^2), ... % Sphere function

@(x) sum(100*(x(2:end)-x(1:end-1).^2).^2 + (1-x(1:end-1)).^2), ... % Rosenbrock function

@(x) -20*exp(-0.2*sqrt(sum(x.^2)/numel(x))) - exp(sum(cos(2*pi*x))/numel(x)) + 20 + exp(1)}; % Ackley function

function_names = {'Sphere', 'Rosenbrock', 'Ackley'};

% Dimensions to test

dimensions = [2, 10];

% Number of runs

num_runs = 15;

% Optimization algorithms to compare

algorithms = {@simple_ga, @simple_pso, @simple_sa};

alg_names = {'Simple Genetic Algorithm', 'Simple Particle Swarm Optimization', 'Simple Simulated Annealing'};

% Results storage

results = struct();

% Convergence history storage

convergence_history = cell(length(dimensions), length(functions), length(algorithms));

% Create 3D surface plots for each function

create_3d_plots(functions, function_names);

for d = 1:length(dimensions)

D = dimensions(d);

for f = 1:length(functions)

func = functions{f};

func_name = function_names{f};

% Set bounds for optimization

lb = -100 * ones(1, D);

ub = 100 * ones(1, D);

for a = 1:length(algorithms)

alg = algorithms{a};

alg_name = alg_names{a};

fprintf('Testing %s on %s (D=%d)\n', alg_name, func_name, D);

% Run optimization multiple times

fitness_values = zeros(1, num_runs);

for run = 1:num_runs

[~, fval, history] = alg(func, D, lb, ub);

fitness_values(run) = fval;

% Store convergence history for the first run

if run == 1

convergence_history{d, f, a} = history;

end

end

% Store results

results(d,f,a).algorithm = alg_name;

results(d,f,a).function = func_name;

results(d,f,a).dimension = D;

results(d,f,a).mean = mean(fitness_values);

results(d,f,a).std = std(fitness_values);

results(d,f,a).best = min(fitness_values);

results(d,f,a).worst = max(fitness_values);

end

end

end

% Display results

for d = 1:length(dimensions)

for f = 1:length(functions)

fprintf('\nResults for %s (D=%d):\n', function_names{f}, dimensions(d));

for a = 1:length(algorithms)

r = results(d,f,a);

fprintf('%s:\n', r.algorithm);

fprintf('  Mean: %.4e (±%.4e)\n', r.mean, r.std);

fprintf('  Best: %.4e\n', r.best);

fprintf('  Worst: %.4e\n', r.worst);

end

end

end

% Plot convergence

for d = 1:length(dimensions)

for f = 1:length(functions)

figure;

hold on;

for a = 1:length(algorithms)

history = convergence_history{d, f, a};

plot(1:length(history), history, 'DisplayName', alg_names{a});

end

xlabel('Iterations');

ylabel('Fitness Value');

title(sprintf('Convergence for %s (D=%d)', function_names{f}, dimensions(d)));

legend('show');

set(gca, 'YScale', 'log');  % Use log scale for y-axis

hold off;

end

end

% Function to create 3D surface plots

function create_3d_plots(functions, function_names)

[X, Y] = meshgrid(-5:0.1:5, -5:0.1:5);

for i = 1:length(functions)

Z = zeros(size(X));

for j = 1:size(X, 1)

for k = 1:size(X, 2)

Z(j, k) = functions{i}([X(j, k), Y(j, k)]);

end

end

figure;

surf(X, Y, Z);

title(['3D Surface Plot of ', function_names{i}, ' Function']);

xlabel('x');

ylabel('y');

zlabel('z');

end

end

% Simple Genetic Algorithm

function [best_solution, best_fitness, history] = simple_ga(func, D, lb, ub)

pop_size = 50;

num_generations = 100;

mutation_rate = 0.1;

population = lb + (ub - lb) .* rand(pop_size, D);

fitness = zeros(pop_size, 1);

history = zeros(1, num_generations);

for gen = 1:num_generations

for i = 1:pop_size

fitness(i) = func(population(i, :));

end

[best_fitness, idx] = min(fitness);

best_solution = population(idx(1), :);

history(gen) = best_fitness;

% Select top 2 individuals (or clone the best if only one unique fitness value)

[~, sorted_idx] = sort(fitness);

if length(unique(fitness)) == 1

new_pop = repmat(population(sorted_idx(1), :), 2, 1);

else

new_pop = population(sorted_idx(1:2), :);

end

while size(new_pop, 1) < pop_size

parents = population(randperm(pop_size, 2), :);

child = mean(parents, 1);

if rand < mutation_rate

child = child + randn(1, D) .* (ub - lb) * 0.1;

end

child = max(min(child, ub), lb);  % Ensure child is within bounds

new_pop = [new_pop; child];

end

population = new_pop;

end

end

% Simple Particle Swarm Optimization

function [best_solution, best_fitness, history] = simple_pso(func, D, lb, ub)

num_particles = 50;

num_iterations = 100;

w = 0.7;

c1 = 1.4;

c2 = 1.4;

positions = lb + (ub - lb) .* rand(num_particles, D);

velocities = zeros(num_particles, D);

personal_best_pos = positions;

personal_best_fit = inf(num_particles, 1);

global_best_pos = zeros(1, D);

global_best_fit = inf;

history = zeros(1, num_iterations);

for iter = 1:num_iterations

for i = 1:num_particles

fitness = func(positions(i, :));

if fitness < personal_best_fit(i)

personal_best_fit(i) = fitness;

personal_best_pos(i, :) = positions(i, :);

end

if fitness < global_best_fit

global_best_fit = fitness;

global_best_pos = positions(i, :);

end

end

history(iter) = global_best_fit;

for i = 1:num_particles

r1 = rand(1, D);

r2 = rand(1, D);

velocities(i, :) = w * velocities(i, :) + ...

c1 * r1 .* (personal_best_pos(i, :) - positions(i, :)) + ...

c2 * r2 .* (global_best_pos - positions(i, :));

positions(i, :) = positions(i, :) + velocities(i, :);

positions(i, :) = max(min(positions(i, :), ub), lb);

end

end

best_solution = global_best_pos;

best_fitness = global_best_fit;

end

% Simple Simulated Annealing

function [best_solution, best_fitness, history] = simple_sa(func, D, lb, ub)

max_iter = 1000;

T0 = 100;

Tf = 1e-8;

alpha = 0.95;

current_solution = lb + (ub - lb) .* rand(1, D);

current_energy = func(current_solution);

best_solution = current_solution;

best_fitness = current_energy;

T = T0;

history = zeros(1, max_iter);

for i = 1:max_iter

neighbor = current_solution + randn(1, D) .* (ub - lb) * 0.1;

neighbor = max(min(neighbor, ub), lb);

neighbor_energy = func(neighbor);

if neighbor_energy < current_energy || rand < exp((current_energy - neighbor_energy) / T)

current_solution = neighbor;

current_energy = neighbor_energy;

end

if current_energy < best_fitness

best_solution = current_solution;

best_fitness = current_energy;

end

history(i) = best_fitness;

T = T0 * alpha^i;

if T < Tf

break;

end

end

history = history(1:i);  % Trim unused elements if SA terminates early

end

OUTPUT FOR THE TASK 2:

Testing Simple Genetic Algorithm on Sphere (D=2)

Testing Simple Particle Swarm Optimization on Sphere (D=2)

Testing Simple Simulated Annealing on Sphere (D=2)

Testing Simple Genetic Algorithm on Rosenbrock (D=2)

Testing Simple Particle Swarm Optimization on Rosenbrock (D=2)

Testing Simple Simulated Annealing on Rosenbrock (D=2)

Testing Simple Genetic Algorithm on Ackley (D=2)

Testing Simple Particle Swarm Optimization on Ackley (D=2)

Testing Simple Simulated Annealing on Ackley (D=2)

Testing Simple Genetic Algorithm on Sphere (D=10)

Testing Simple Particle Swarm Optimization on Sphere (D=10)

Testing Simple Simulated Annealing on Sphere (D=10)

Testing Simple Genetic Algorithm on Rosenbrock (D=10)

Testing Simple Particle Swarm Optimization on Rosenbrock (D=10)

Testing Simple Simulated Annealing on Rosenbrock (D=10)

Testing Simple Genetic Algorithm on Ackley (D=10)

Testing Simple Particle Swarm Optimization on Ackley (D=10)

Testing Simple Simulated Annealing on Ackley (D=10)

Results for Sphere (D=2):

Simple Genetic Algorithm:

Mean: 1.4624e-02 (±1.9850e-02)

Best: 1.2906e-04

Worst: 7.3739e-02

Simple Particle Swarm Optimization:

Mean: 5.4667e-12 (±6.9601e-12)

Best: 1.4738e-13

Worst: 2.4972e-11

Simple Simulated Annealing:

Mean: 1.3228e+00 (±1.2321e+00)

Best: 9.5640e-02

Worst: 4.1638e+00

Results for Rosenbrock (D=2):

Simple Genetic Algorithm:

Mean: 1.2138e+00 (±3.5358e+00)

Best: 1.9321e-04

Worst: 1.3904e+01

Simple Particle Swarm Optimization:

Mean: 8.6764e+00 (±2.3274e+01)

Best: 1.4238e-07

Worst: 7.9126e+01

Simple Simulated Annealing:

Mean: 4.0320e+01 (±3.8652e+01)

Best: 1.4679e-02

Worst: 1.2542e+02

Results for Ackley (D=2):

Simple Genetic Algorithm:

Mean: 5.2961e-01 (±6.7402e-01)

Best: 5.5399e-02

Worst: 2.4639e+00

Simple Particle Swarm Optimization:

Mean: 1.5310e-05 (±3.6214e-05)

Best: 6.7158e-07

Worst: 1.4464e-04

Simple Simulated Annealing:

Mean: 1.1164e+01 (±7.2547e+00)

Best: 1.5165e+00

Worst: 2.0000e+01

Results for Sphere (D=10):

Simple Genetic Algorithm:

Mean: 1.7861e+02 (±1.2570e+02)

Best: 3.8571e+01

Worst: 4.7625e+02

Simple Particle Swarm Optimization:

Mean: 2.6056e-04 (±2.2351e-04)

Best: 3.7404e-05

Worst: 8.7041e-04

Simple Simulated Annealing:

Mean: 8.9797e+02 (±1.9746e+02)

Best: 6.1791e+02

Worst: 1.2555e+03

Results for Rosenbrock (D=10):

Simple Genetic Algorithm:

Mean: 2.4031e+05 (±3.0497e+05)

Best: 2.1467e+04

Worst: 1.0296e+06

Simple Particle Swarm Optimization:

Mean: 2.0014e+05 (±4.1397e+05)

Best: 7.9673e+00

Worst: 1.0000e+06

Simple Simulated Annealing:

Mean: 1.3675e+07 (±6.5277e+06)

Best: 4.1877e+06

Worst: 3.0301e+07

Results for Ackley (D=10):

Simple Genetic Algorithm:

Mean: 1.1587e+01 (±1.8613e+00)

Best: 8.9594e+00

Worst: 1.5076e+01

Simple Particle Swarm Optimization:

Mean: 2.0017e+01 (±4.1512e-02)

Best: 2.0000e+01

Worst: 2.0155e+01

Simple Simulated Annealing:

Mean: 2.0478e+01 (±1.5931e-01)

Best: 2.0237e+01

Worst: 2.0827e+01

AND GRAPHS OF TASK 2: